Research Proposal

Proposed Title

"ITU-T G.711 Standard Compliance Gap in Deep Learning Voice Activity Detection: A Domain Adaptation Study with Embedded I2S Interface Validation"

1. Problem Statement

Voice Activity Detection (VAD) is a foundational component of telephony communication systems, directly governed by the ITU-T G.711 standard — the most widely deployed telephony codec worldwide. The G.711 standard specifies 8kHz sampling and A-law logarithmic companding, creating a signal processing domain fundamentally different from the 16kHz clean audio used to train all existing deep learning VAD models.

The critical gap: No existing work has studied whether state-of-the-art VAD models comply with the signal characteristics imposed by the ITU-T G.711 standard. This paper fills that gap with the first systematic empirical study.

2. Experimental Evidence

We evaluated three state-of-the-art off-the-shelf VAD systems on G.711 A-law encoded audio:

Table 1: Off-the-Shelf VAD Performance on G.711 A-law Data

| Model | Design Target | Parameters | Accuracy | F1 |
| --- | --- | --- | --- | --- |
| Silero VAD | General purpose, 6000+ languages | ~1M | 49.90% | 0.411 |
| WebRTC VAD | Telephony-specific, 8kHz | N/A | 49.00-50.00% | 0.333 |
| Random baseline | — | — | 50.00% | 0.333 |

The results are unambiguous. Both models — including WebRTC VAD which was explicitly designed for 8kHz telephony — perform at random guessing level on G.711 A-law data. This represents a critical standards compliance failure: systems deployed in ITU-T G.711 telephony environments cannot rely on existing off-the-shelf VAD.

3. Proposed Solution and Results

We propose a G.711-compliant VAD training framework consisting of:

Component 1 — G.711 A-law simulation pipeline: A software simulation of the complete G.711 codec path validated against real RTP network traffic. The simulation achieves 100% byte-level agreement with real G.711 PCMA packets (KL divergence = 0.000), confirming mathematical equivalence to hardware codec output.

Component 2 — Domain-matched training: Four lightweight neural network architectures trained on G.711-processed audio, enabling standard-compliant VAD without requiring physical codec hardware during training.

Component 3 — I2S interface validation: All models deployed on Raspberry Pi Compute Module 5 (CM5) connected to a TP3094 Quad PCM Codec via I2S interface — the exact hardware configuration of our production telephony system.

Table 2: G.711-Compliant VAD Results

| Model | Parameters | Size | Accuracy | F1 | CM5 Latency | Real-time? |
| --- | --- | --- | --- | --- | --- | --- |
| CNN1D | 124K | 488KB | 97.97% | 0.9797 | 0.307ms | ✓ |
| WaveNetSmall | 175K | 686KB | 99.42% | 0.9942 | 0.764ms | ✓ |
| ECAPAVAD | 242K | 960KB | 99.65% | 0.9965 | 2.334ms | ✓ |
| TransformerVAD | 71K | 344KB | 97.80% | 0.9780 | 0.466ms | ✓ |
| Silero (baseline) | ~1M | ~1800KB | 49.90% | 0.411 | — | — |
| WebRTC (baseline) | N/A | N/A | 49-50% | 0.333 | — | — |

All models run in real-time on CM5 ARM hardware — the 32ms constraint imposed by 256-sample windows at 8kHz is satisfied with 10-100x margin.

4. Key Contributions — Framed for CS&I

Contribution 1 — Standards compliance gap identification: First systematic demonstration that ITU-T G.711 A-law companding causes catastrophic domain mismatch in deep learning VAD (49-50% accuracy), affecting both general-purpose and telephony-specific off-the-shelf systems.

Contribution 2 — G.711-compliant simulation framework: A validated software simulation of the ITU-T G.711 A-law codec path enabling standard-compliant VAD training. Validated at 100% byte-level agreement with real G.711 PCMA RTP network traffic — a reproducible contribution that allows any researcher to generate G.711-compliant training data.

Contribution 3 — I2S interface deployment validation: Complete validation of the proposed VAD system on real embedded I2S hardware (CM5 + TP3094 + I2S), measuring real-time inference latency on ARM Cortex-A76 CPU using ONNX runtime. All models satisfy the 32ms real-time constraint with 10-100x margin.

Contribution 4 — Lightweight standard-compliant models: Four neural architectures (71K-242K parameters, 344-960KB) achieving 97-99% accuracy on G.711 A-law data — suitable for resource-constrained embedded telephony systems governed by ITU-T standards.

5. Validation Evidence

Simulation validation figure: Statistical comparison of software simulation against real G.711 PCMA RTP traffic shows perfect agreement (KL divergence = 0.000, byte match = 100%).

Hardware deployment: All four models successfully deployed on CM5 via ONNX runtime with measured latency of 0.307ms to 2.334ms — within the 32ms real-time constraint for 8kHz telephony processing.

6. Current Status

Completed:

✓ G.711 A-law simulation pipeline (validated)

✓ 4 model architectures trained (2 conditions)

✓ Off-the-shelf baselines (Silero + WebRTC)

✓ CM5 hardware deployment (latency measured)

✓ Statistical evaluation (accuracy, F1, precision, recall)

Pending:

□ Paper writing

□ Visualization figures

□ Final revision and submission

Summary

"We identify and resolve a critical compliance gap between the ITU-T G.711 telephony standard and existing deep learning VAD systems. Both Silero VAD and WebRTC VAD — the two leading off-the-shelf VAD solutions — achieve only 49-50% accuracy on G.711 A-law encoded audio, equivalent to random guessing. We propose a G.711-compliant training framework validated at 100% byte-level agreement with real RTP network traffic, enabling four lightweight neural architectures (71K-242K parameters) to achieve 97-99% accuracy. All models are validated on real I2S embedded hardware (CM5 + TP3094), achieving 0.307ms to 2.334ms inference latency — 10-100x within the 32ms real-time constraint. This is, to our knowledge, the first work to systematically study ITU-T G.711 standard compliance in deep learning VAD with embedded hardware validation."